Table of contents

# SD-WAN vs. DIA Under TIC 3.0

Transport is a physics decision; control is a compliance decision

## Problem, solution, ask

Problem. Federal network programs keep framing “SD-WAN vs. DIA” as a choice. Direct Internet Access is a pipe that solves the geographic hairpin; SD-WAN is the control surface that governs pipes. Treated as alternatives, agencies end up with a governed network too slow for voice or a fast network that is a compliance finding — and a bare DIA branch egress sits outside any assessed boundary.

Solution. Transport is a physics decision (fiber, MPLS, LEO satellite, and 5G differ enormously in physics and close exactly one control family: telecommunications diversity). Control is a compliance decision (the SD-WAN/SASE governor closes the protection controls). TIC 3.0 is why you cannot stop at the pipe — and the governor is itself attack surface.

Ask. Before the next transport or SD-WAN procurement: run the decision sequence against the current portfolio, file the authorization-boundary update before cutover rather than after, and name an owner for each step.

## The false versus

DIA is a pipe. SD-WAN is the control surface that governs pipes. DIA solves exactly one problem — the geographic hairpin — and satisfies none of the confidentiality, integrity, flow-enforcement, boundary, malware, or monitoring controls claimed near it. Deployed bare, it leaves the branch egress outside any assessed boundary.

## The physics

Light in fiber travels at roughly 200,000 km/s. ITU-T G.114 budgets 150 ms one-way for acceptable conversational quality. A field office hairpinning through a distant data center spends 100–130 ms one-way on the detour. Local breakout deletes the propagation and routing-inflation terms and moves inspection to where the latency budget can afford it. No security product shortens a fiber path.

Every transport — fiber DIA, MPLS, LEO satellite, 5G — is a pipe. They differ enormously in physics and not at all in their contribution to the protection controls, which is zero for all of them. What transport selection does close is telecommunications-services diversity.

## TIC 3.0 as the policy gate

TIC 3.0 replaced mandatory consolidation with a use-case model: branch traffic may egress locally only when the security-capabilities catalog is satisfied at a distributed policy enforcement point — and only while the agency’s telemetry obligations are met. The policy enforcement point also has a location, and the physics argument applies to it.

## SD-WAN as the transport governor

SD-WAN contributes circuit-wide encryption, traffic-class separation, and application-layer path telemetry, plus active-active multipath. The boundary, flow, and malware closures come from the SASE stack above it. SD-WAN alone does not satisfy the TIC 3.0 Branch Office use case.

## The governor is itself attack surface

A controller with agency-wide routing and policy authority, and a fleet of internet-facing edges replacing two hardened egress points, are consequential new exposures. Edge-management-plane products are among the most actively exploited categories in federal networks. A paper that credits the governor with closures owes the reader the controls that protect the governor.

## Decision sequence

- Buy the pipe on measured physics within the acquisition and supply-chain envelope.

- Deploy the governor before production traffic.

- Treat the boundary change as the significant change it is.

- Handle Microsoft 365 traffic through its own documented exception rather than an ad hoc workaround.

- Register the supporting technical evidence.

Doing these out of order — especially lighting up fast connections before the security layer is live — is the single most common and most costly mistake.

Cleaned extraction for project use. No internal branding or document identifiers.